l�#@l�#@ screen.

First, install theese three good packages:

-less (a pager I like for watching man pages more confortable)

-bzip2 (for uncompressing linrad *tar.bz)

-links ( a good text mode WEB browser for browsing the net)

#apt-get install less bzip2 links

Then install the gcc compiler and all related stuff for compiling:

#apt-get install build-essential

This should take some time (27.8MB) After that, all the great compiling

software from the GNU is ready.

Now, lets download svgalib and linrad. If you do not know where to

download them, use the "links" text mode WEB browser and google for it!

To retrieve the files, use the great "wget" text mode downloading

program. It allows resuming broken downloads. Download the Svgalib and

Linrad files on your home directory: (note that if your card is

supported by the old svgalib 1.4.3 that is shipped with Debian, you do

not need to compile the 1.9.20, but apt-get install the packaged version)

go to your home directory:

$ cd

download the files:

$ wget -c http://www.nitehawk.com/sm5bsz/linuxdsp/archive/lir01-33.tbz

and

$ wget -c http://www.arava.co.il/matan/svgalib/svgalib-1.9.20.tar.gz

If you are using this guide and new Linrad and svgalib are released, you

can use the "links" text mode WEB browser to surf the Internet and

search for the new file locations.

For example

$ links http://www.google.com

will start the links text mode browser.

Install nasm package from the debian servers:

# apt-get install nasm

Now, there is a little issue that I found. For compiling svgalib_helper

module, we need the kernel headers. So we will install the package

kernel-headers-2.6.8-2-386. I do not understand why there is NOT a

kernel-headers-2.6.8-1-386 file, to mach the kernel-image-2.6.8-1-386

that was installed by Debian. So, I also installed the matching kernel

image.

install the headers:

# apt-get install kernel-headers-2.6.8-2-386

and the kernel image that matches with the headers:

#apt-get install kernel-image-2.6.8-2-386

when it asks you "do you want to stop now?--->  SAY "N" and continue.

NOTE: As I do not use any boot loader for my experimental debian setup

(I boot

if from my other Debian machine) , I can not confirm it, but the new

kernel-image install should modify your grub setting to point to the new

kernel version. If it is not the case, modify /boot/grub/menu.1st to

point to the new kernel image and initrd image version.

(I would like that someone could verify this)

Now reboot to use the new kernel:

# reboot

When booted, verify that the command:

$ uname -a

gives the 2.6.8-2 version.

Now it is time to install ALSA stuff:

#apt-get install alsa-utils alsa-oss

Note that alsa-utils will install alsa-base as a dependency. Alsa-base

will prevent from loading the OSS and ALSA drivers at the same

time. (debian uses the "hotplug" and "discover" to do it at boot time).

When installing alsa-base it modifies the "discover" behaviour to

exclude the OSS sound drivers. /etc/discover.d/)

Reboot again just to see everything works.

(This could be really not needed, but it is better to do it to check

that OSS drivers are unloaded. In my case, before installing ALSA, there

were OSS and ALSA drivers for my sound card loaded at the same time.

# reboot

Configure the sound card with the "alsaconf" utility. It is very

intuitive to choose your sound card. It will detect it automagically.:

# alsaconf

We are ready to compile svgalib:

Go to the directory in with you have downloaded the sources.

# tar xfvz svgalib-1.9.20

# cd svgalib-1.9.20

# make install

# make demoprogs

# cd demos

# depmod -a

load the svgalib_helper module manually:

# modprobe svgalib_helper

run the test program to see if svgalib works:

# ./vgatest

if it does not work, you should edit /etc/vga/libvga.config file to match

your hardware, but It should do it automatically.

lets test your mouse setup:

#./mousetest

and move your mouse and click  the buttons to see if it works

If it does not work, you have to edit the config file

/etc/vga/libvga.config and choose your mouse type and mouse device. In

my case, I commented "mouse Microsoft" and  uncommented "mouse PS2". I

also changed "/dev/ttyS0" for "/dev/psaux" to point my right mouse PS2

device.

Now it is ttime to compile Linrad:

Go to your home directory:

$ cd

uncompress the source:

# tar xfvj lir01-33.tbz

# cd linrad-01.33

# ./configure

# make

Note that svgalib_helper module was loaded before.

if not, you can use

#modprobe svgalib_helper

to load it

Run linrad

# ./linrad

and follow Leif directions.

#rmmod svgalib_helper

will unload that module from memory.

You can write a simple bash script and locate it on your linrad

directory. Call it for example, "runlinrad" and "chmod 755 runlinrad" to

make it executable.

************************

#!/bin/bash

# script to load module, run linrad, and unload module.

modprobe svgalib_helper

./linrad

rmmod svgalib_helper

************************

So you only need to invoque that script to run Linrad:

./runlinrad

That is all.

Please send me any comments, corrections or suggestions. I will also

appreciate that you correct my english ;-)

Leif, if possible, I would like to see this guide published somewhere in

your web page to help Debian hams to setup Linrad.

Thank you very much.

73, Ramiro. EA1ABZ.

#############################################################

This message is sent to you because you are subscribed to

the mailing list <linrad@antennspecialisten.se>.

To unsubscribe, E-mail to: <linrad-off@antennspecialisten.se>

To switch to the DIGEST mode, E-mail to <linrad-digest@antennspecialisten.se>

To switch to the INDEX mode, E-mail to <linrad-index@antennspecialisten.se>

Send administrative queries to  <linrad-request@antennspecialisten.se>�7##�###Hello friends.

I have just installed Linrad perfectly in my new debian experimental

setup. I have written this little guide, in which it is show how to

setup svgalib and Linrad in a basic debian text machine.

I asume that you have just installed Debian Sarge with the rc-3

installer. I also asume  (it was my case) that the Debian installation

was made without internet access and that you have a modem. If you

installed Debian trough the Internet by other methods, you should have

an Internet connection running. Lets go!

Configure the internet access:

#apt-get install pppconfig

#pppconfig

and answer the questions to create a new Internet connection. If you

want to be able to start the internet session as a normal user (not only

as root), do not forget to select in the advanced options: "add user"

to start the modem connection, send the command (you do not need to be

root):

$ pon

Well, we are on the Internet.

Now, configure the mirrors from wich apt will download the files:

# apt-setup

choose the desired ftp mirrors on the dialog screen.

First, install theese three good packages:

-less (a pager I like for watching man pages more confortable)

-bzip2 (for uncompressing linrad *tar.bz)

-links ( a good text mode WEB browser for browsing the net)

#apt-get install less bzip2 links

Then install the gcc compiler and all related stuff for compiling:

#apt-get install build-essential

This should take some time (27.8MB) After that, all the great compiling

software from the GNU is ready.

Now, lets download svgalib and linrad. If you do not know where to

download them, use the "links" text mode WEB browser and google for it!

To retrieve the files, use the great "wget" text mode downloading

program. It allows resuming broken downloads. Download the Svgalib and

Linrad files on your home directory: (note that if your card is

supported by the old svgalib 1.4.3 that is shipped with Debian, you do

not need to compile the 1.9.20, but apt-get install the packaged version)

go to your home directory:

$ cd

download the files:

$ wget -c http://www.nitehawk.com/sm5bsz/linuxdsp/archive/lir01-33.tbz

and

$ wget -c http://ww	\##\�#@\�#@l/matan/svgalib/svgalib-1.9.20.tar.gz

If you are using this guide and new Linrad and svgalib are released, you

can use the "links" text mode WEB browser to surf the Internet and

search for the new file locations.

For example

$ links http://www.google.com

will start the links text mode browser.

Install nasm package from the debian servers:

# apt-get install nasm

Now, there is a little issue that I found. For compiling svgalib_helper

module, we need the kernel headers. So we will install the package

kernel-headers-2.6.8-2-386. I do not understand why there is NOT a

kernel-headers-2.6.8-1-386 file, to mach the kernel-image-2.6.8-1-386

that was installed by Debian. So, I also installed the matching kernel

image.

install the headers:

# apt-get install kernel-headers-2.6.8-2-386

and the kernel image that matches with the headers:

#apt-get install kernel-image-2.6.8-2-386

when it asks you "do you want to stop now?--->  SAY "N" and continue.

NOTE: As I do not use any boot loader for my experimental debian setup

(I boot

if from my other Debian machine) , I can not confirm it, but the new

kernel-image install should modify your grub setting to point to the new

kernel version. If it is not the case, modify /boot/grub/menu.1st to

point to the new kernel image and initrd image version.

(I would like that someone could verify this)

Now reboot to use the new kernel:

# reboot

When booted, verify that the command:

$ uname -a

gives the 2.6.8-2 version.

Now it is time to install ALSA stuff:

#apt-get install alsa-utils alsa-oss

Note that alsa-utils will install alsa-base as a dependency. Alsa-base

will prevent from loading the OSS and ALSA drivers at the same

time. (debian uses the "hotplug" and "discover" to do it at boot time).

When installing alsa-base it modifies the "discover" behaviour to

exclude the OSS sound drivers. /etc/discover.d/)

Reboot again just to see everything works.

(This could be really not needed, but it is better to do it to check

that OSS drivers are unloaded. In my case, before installing ALSA, there

were OSS and ALSA drivers for my sound card loaded at the same time.

# reboot

Configure the sound card with the "alsaconf" utility. It is very

intuitive to choose your sound card. It will detect it automagically.:

# alsaconf

We are ready to compile svgalib:

Go to the directory in with you have downloaded the sources.

# tar xfvz svgalib-1.9.20

# cd svgalib-1.9.20

# make install

# make demoprogs

# cd demos

# depmod -a

load the svgalib_helper module manually:

# modprobe svgalib_helper

run the test program to see if svgalib works:

# ./vgatest

if it does not work, you should edit /etc/vga/libvga.config file to match

your hardware, but It should do it automatically.

lets test your mouse setup:

#./mousetest

and move your mouse and click  the buttons to see if it works

If it does not work, you have to edit the config file

/etc/vga/libvga.config and choose your mouse type and mouse device. In

my case, I commented "mouse Microsoft" and  uncommented "mouse PS2". I

also changed "/dev/ttyS0" for "/dev/psaux" to point my right mouse PS2

device.

Now it is ttime to compile Linrad:

Go to your home directory:

$ cd

uncompress the source:

# tar xfvj lir01-33.tbz

# cd linrad-01.33

# ./configure

# make

Note that svgalib_helper module was loaded before.

if not, you can use

#modprobe svgalib_helper

to load it

Run linrad

# ./linrad

and follow Leif directions.

#rmmod svgalib_helper

will unload that module from memory.

You can write a simple bash script and locate it on your linrad

directory. Call it for example, "runlinrad" and "chmod 755 runlinrad" to

make it executable.

************************

#!/bin/bash

# script to load module, run linrad, and unload module.

modprobe svgalib_helper

./linrad

rmmod svgalib_helper

************************

So you only need to invoque that script to run Linrad:

./runlinrad

That is all.

Please send me any comments, corrections or suggestions. I will also

appreciate that you correct my english ;-)

Leif, if possible, I would like to see this guide published somewhere in

your web page to help Debian hams to setup Linrad.

Thank you very much.

73, Ramiro. EA1ABZ.

#############################################################

This message is sent to you because you are subscribed to

the mailing list <linrad@antennspecialisten.se>.

To unsubscribe, E-mail to: <linrad-off@antennspecialisten.se>

To switch to the DIGEST mode, E-mail to <linrad-digest@antennspecialisten.se>

To switch to the INDEX mode, E-mail to <linrad-index@antennspecialisten.se>

Send administrative queries to  <linrad-request@antennspecialisten.se>

#ennspecialisten.se>

To switch to the DIGEST mode, E-mail to <linrad-digest@antennspecialisten.se>

To switch to the INDEX mode, E-mail to <linrad-index@antennspecialisten.se>

Send administrative queries to  <linrad-request@antennspecialisten.se>

#-y#�## (you must have root

>>> privileges do to this !)

>>> Check that you have a symbolic link called build in the

>>> /lib/modules/2.6.x-y-zzz directory.

>>> Ok, now we go back to leif's installation procedure :

>>> cd /usr/src                     ( go to /usr/src )

>>> mkdir svgalib                   ( create a svgalib directory)

>>> cd svgalib                      ( go to that directory)

>>> tar -xvzf /home/<your_user_name>/svgalib-1.9.19.tar.gz

>>> cd svgalib-1.9.19               (this may take a while !)

>>> You have installed svgalib! If your system is new,you need to install

>>> nasm with :

>>> apt-get install nasm

>>> now, we can install linrad itself :

>>> cd /home                        (goto your home directory)

>>> tar -xvjf <your_user_name>/lir01-33.tbz

>>>                                 ( this creates a linrad-01.33 folder )

>>> cd linrad-01.33                 ( go to that directory )

>>> ./configure                     (check dependencies and build

>>> makefile) make                            ( compile linrad )

>>> make svgalib            ( install svgalib_helper at boot and /dev/svga )

>>> cd /etc/vga

>>> vi libvga.config                ( edit libvga config file )

>>> edit the line beginning  by mouse to select the mouse type, most of us

>>> need : mouse PS2

>>> add a line containing

>>> mdev /dev/input/mice

>>> :wq                             ( save modification and exit editor )

>>> Now is a good time to reboot your system (this will load svgalib_helper)

>>> open a terminal with root privileges :

>>> cd /home/linrad-01.33           ( go to linrad's directory )

>>> ./vgatest                       ( test svgalib config )

>>> ./linrad                        ( enjoy linrad )

>>>

>

>

>

#############################################################

This message is sent to you because you are subscribed to

the mailing list <linrad@antennspecialisten.se>.

To unsubscribe, E-mail to: <linrad-off@antennspecialisten.se>

To switch to the DIGEST mode, E-mail to <linrad-digest@antennspecialisten.se>

To switch to the INDEX mode, E-mail to <linrad-index@antennspecialisten.se>

Send administrative queries to  <linrad-request@antennspecialisten.se>

#@antennspecialisten.se>

To switch to the INDEX mode, E-mail to <linrad-index@antennspecialisten.se>

Send administrative queries to  <linrad-request@antennspecialisten.se>

#to woѫ##

> >

> > Thanks and 73,

> >

> > Roger

> > W3SZ

> >

> >

>

> #sin��##or linrad usage, you should

>>really sweep a sine generator from zero to beyond the Nyquist

>>frequency of a given converter to determine its aliasing behaviour.

>>

>>Of course, none of this matters if your receiver's (analog) RF/IF/AF

>>filters are steeper and/or narrower than your ADC's anti-aliasing

>>filter...

>>

>>By the way, I am thinking about doing a backend for a linrad/SDR-1000

>>like receiver with A/D converters integrated in the final stage

>>(behind the I/Q demodulator). Not only does this allow to put the ADC

>>in the mast (running S/PDIF-like signals into the shack over Cat5 or

>>fiber), but it also allows hard synchronisation between the sample

>>clock and other receiver clocks, reducing spurs and drift.

>>

>>Regards,

>>

>>JDB.

>>

>>[1] Y#<##L�#@L�#@ 96kHz (-48...+48kHz complex bandwidth) from a

>>stereo-sampled I/Q signal on a 96kHz soundcard, but only if you're

>>either willing to accept aliasing noise at the band edges, or if you

>>can build a dual tracking anti-aliasing filter sharper than the sound

>>chip's digital filter (think > 250dB/octave for a good converter).

>>

>>[2] And even 'sampling rate' is a misnomer with modern audio ADCs,

>>seeing that they all use some form of sigma-delta conversion, which

>>(very simply said) is an oversampling lower-order ADC (someta�##

>>1-bit, sometimes multibit) inside a feedback loop, combined with a

>>digital filter/decimator to produce the output samples.

>>--

>>LART. 250 MIPS under one Watt��##ee hardware design files.

>>http://www.lart.tudelft.nl/

>>

>>

>>    q�##>

>

i�##

>#ples.

>>--

>>LART. 250 MIPS under one Watt. Free hardware design files.

>>http://www.lart.tudelft.nl/

>>

>>

>>

>>

>

>

>

>

>#Li��##

>> really

>> and [2] I didn's spend enough time on it [only wasted an evening], what

>> it

>> shows to me is #h d###"##y updatedb, to

> make lookups speedy.

>

> If you remove the slocate package, you lose the ability to find files in

> a fast way, and have to use the slower find command.

>

> You could also remove the /etc/cron.daily/slocate.cron, or move it out

> of that directory, say to /root/bin/ and then manually run slocate.cron

> wh#n pe##�"## by the mean of the 'Mozilla Web

> Browser' (see Application - Internet)

>

> You should download the following files to your persona# f#f###### by #he #f###### by the mean of the 'Mozilla Web

> Browser' (see Application - Internet)

>

> You should download the following files to your personal folder

> (/home/<your_user_name>) :

> - lir01-33.tbz (linrad itself)

> - svgalib-1.9.19.tar.gz (sri , I haven't tried 1.9.20 yet !)

>

> (there is no need to download 'nasm' as we can to this with an 'apt-

> get')

>

> Installing components :

>

> First, we need to install svgalib but it won't compile unless we install

> a kernel-header package. The command 'apt-cache search kernel-headers'

> will give you all the available packages.If you do not know which one

> should be used do this :

>

> Switch to the text console #1 (type Ctrl - ALT - F1)

> You should see a prompt asking a login : type root

> Now the password (Type the password of the superuser)

> On the first line, you should see : Linux Debian 2.6.x-y-zzz

> this is the exact version of your kernel. In my case, I have 2.6.8-2-386

> so in the following, replace the x-y-zzz by the appropriate numbers.

> BTW, you can go back to the graphical mode with Ctrl-Alt-F7

>

> Intall the kernel-header with the folowing command :

> apt-get install kernel-headers-2.6.x-y-zzz

> (you must have root privileges do to th��##)

>

> Check that you have a symbolic link called build in the

> /lib/modules/2.6.x-y-zzz directory.

>

> Ok, now we go back to leif's installation procedure :

> cd /usr/src			( go to /usr/src )

> mkdir svgalib			( create a svgalib directory)

> cd svgalib			( go to that directory)

> tar -xvzf /home/<your_user_name>/svgalib-1.9.19.tar.gz

> cd svgalib-1.9.19     		(this may take a while !)

>

> You have installed svgalib! If your system is new,you need to install

> nasm with :

> apt-get install nasm

>

> now, we can install linrad itself :

> cd /home			(goto your home directory)

> tar -xvjf <your_user_name>/lir01-33.tbz

> 				( this creates a linrad-01.33 folder )

> cd linrad-01.33 		( go to that directory )

> ./configure			(check dependencies and build makefile)

> make 				( compile linrad )

> make svgalib		( install svgalib_helper at boot and /dev/svga )

>

> cd /etc/vga

> vi libvga.config		( edit libvga config file )

> edit the line beginning  by mouse to select the mouse type, most of us

> need : mouse PS2

> add a line containing

> mdev /dev/input/mice

> :wq				( save modification and exit editor )

>

> Now is a good time to reboot your system (this will load svgalib_helper)

>

> open a terminal with root privileges :

> cd /home/linrad-01.33		( go to linrad's directory )

> ./vgatest			( test svgalib config )

> ./linrad 			( enjoy linrad )

>

#############################################################

This message is sent to you because you are subscribed to

the mailing list <linrad@antennspecialisten.se>.

To unsubscribe, E-mail to: <linrad-off@antennspecialisten.se>

To switch to the DIGEST mode, E-mail to <linrad-digest@antennspecialisten.se>

To switch to the INDEX mode, E-mail to <linrad-index@antennspecialisten.se>

Send administrative queries to  <linrad-request@antennspecialisten.se>

#itch to the INDEX mode, E-mail to <linrad-index@antennspecialisten.se>

Send administrative queries to  <linrad-request@antennspecialisten.se>

#ons du��## the last couple of years.

>>

>>Yesterday I installed Debian-base with the rc3-installer through

>>the network, and it found the PCI-HomePNA-card automatically. The

>>(later) downloaded svgalib-bin package was referred to as 1.4.3-21

>>

>>Next I will expand functionality by trying a linrad 01.33 install.

>>

>>Last week I saw that there was a new svgalib 1.9.20 available.

>>

>>So far I have seen posts only referring to 1.9.19 and earlier.

>>I have had my biggest battles with the 1.9.17 - 1.9.19 helper

>>modules, and indeed I have never succeeded under SuSE/Knoppix.

>>If I remember correctly Gentoo had a distro-matched 1.9.17-r2

>>svgalib package and it worked right away in Gentoo 2004.x

>>

>>This is probably my first posting on the 'Linrad' list, but�u##�*##, that a Linux

> system does not need a reboot like the Win-systems.

No, I think that a reboot is not need. You can always load that module

by hand with the modprobe command.

But maybe there has

> been a change in this too, or I have never understood the details. This

> proves that reading instructions is very important! In summary: I learnt

> a lot of new things and I arrived at the same result as before, for the

> time being. My impression is that soon there is enough routine to set up

> a 'Linrad' system from state zero ("empty hardware") to full operation

> (one way or the other) in less than 100 minutes. Maybe we should make

> this a small "contest" to create more attention/interest for 'Linrad'.

Good idea.

>

>

> Wishing you all a relaxed but productive weekend!  73, "Zaba"  OH1ZAA/2

>

Well, I have just install Debian in a spare partition last night, so

today, If I have free time I will try to install last Linrad and last

svgalib to see.

Just a tip for your X install ( I do not know if knoppix uses the Xfree

or the Xorg Window system, but it will be helpfull anyway, there should

not be too much differences). Run knoppix on it. Copy the XF86Config-4

file to your Debian box. It should work.

Any way, tell me the errors you are having. You also can watch the X log

errors in /var/log/XFree86.0.log. Look for the "EE" errors, it will tell

you the reason of the failure.

73, Ramiro.

>

> At 15:04 15.4.2005 +0200, EA1ABZ wrote:

>

>> Patrick wrote:

>>

>>> Hello Ramiro and All,

>>> Here is how I install linrad on Sarge.

>>> First I used this install CD :

>>> http://cdimage.debian.org/pub/cdimage-testing/sarge_d-i/i386/rc3/sarge-

>>> i386-netinst.iso

>>

>>

>>

>> Oh yes!, this is my prefered method (100MB), as all base system is

>> included on the CD and you do not need to download anything to get a

>> console system running. The business card is even smaller (30 MB), but

>> you need to download more. The mini.iso is even smaller (8MB) and more

>> to download.

>>

>>

>> Congratulations for the install report. It will be very useful for the

>> Debian newbies.

>>

>>

>> Thank you very much.

>>

>> 73, Ramiro.

>>

>> PS: I have just downloaded the rc-3 installer (100MB) and tonight I

>> will install it to test in another partition.

>>

>>>

>>> The netinst CD-Rom is much quicker than the mini CD . BE AWARE that

>>> these single CD-Rom install tools download most of the packages via FTP

>>> so if you do not have 1Mb/s transfer rate, this may be very sloooow.

>>> When using the netinst CD-Rom, the volume control is working right at

>>> the end of the install process (The meSB1bmRl

ciBnY2M6LSkNCg0KVU5JWCBkb2VzIG5vdCBkaXN0aW5ndWlzaCBiaW5hcnkgZnJvbSB0ZXh0IGZp

bGVzLCB3aGljaCBpcyB3aHkgdGhlICJ0IiBvciAiYiIgaXMgaWdub3JlZC4gSSBqdXN0IA0KbG9v

a2VkIGF0IHRoZSBtYW4gcGFnZSBvbiBmb3BlbiBhbmQgaXQgc2F5cyB0aGUgImIiIGlzIHN0cmlj

dGx5IGZvciBjb21wYXRhYmlsaXR5IHdpdGggQU5TSSBDIGFuZCBoYXMgDQpubyBlZmZlY3Qgb24g

YW55IFBPU0lYIG9wZXJhdGluZyBzeXN0ZW0uIEl0IHN1Z2dlc3RzIGFkZGluZyB0aGUgImIiIGlm

IHlvdSBkbyBJTyBvZiBiaW5hcnkgZmlsZXMgb24gDQpub24tVU5JWCBvcGVyYXRpbmcgc3lzdGVt

cy4gSSdtIG5vdCBzdXJlIGlmIG15IHByb2JsZW1zIHdlcmUgb24gYmluYXJ5IG9yIHRleHQgZmls

ZXMsIGJ1dCBJIGhhdmUgc2VlbiANCnRoZW0uIEJ1dCBBVExDIHdyaXRlcyBiaXRtYXBzLCB3aGlj

aCBhcmUgYmluYXJ5IGZpbGVzLCBzbyBwZXJoYXBzIHRoYXQgaXMgd2hhdCBjYXVzZXMgdGhlIHBy

b2JsZW0uDQoNCj4+Pj4yKSBJZiBpdCBjb21waWxlcyBvbiBXaW5kb3plIGFuZCB5b3VyIGZhdm9y

aXRlIExpbnV4IChSZWRoYXQgb3IgDQo+Pj4+d2hhdGV2ZXIpIGRvbid0IGFzc3VtZSBpdCB3aWxs

IGNvbXBpbGUgDQo+Pj4+b24gYWxsIExpbnV4L1VOSVggc3lzdGVtcy4NCj4+Pg0KPj4+T0suIEFj

dHVhbGx5IGl0IGNvbXBpbGVzIHVuZGVyIExpbnV4IG9ubHkuIA0KPj4NCj4+SSdtIGNvbmZ1c2Vk

LiBZb3UgaGF2ZSBXaW5kb3dzIGJpbmFyaWVzLCBidXQgaXQgb25seSBjb21waWxlcyB1bmRlciBM

aW51eC4NCj4+UGVyaGFwcyB5b3UgdXNlIEN5Z3dpbj8NCj4gDQo+IE5vLCBJIGhhdmUgb25lIGNv

bXB1dGVyIHdpdGggRGViaWFuIExpbnV4LiBPbiB0aGlzIG1hY2hpbmUgSSBoYXZlDQo+IHN2Z2Fs

aWIsIG5hc20gYW5kIG1pbmd3MzIuIEp1c3QgYnkgdHlwaW5nIG1ha2UgImxpbnJhZC5leGUiIHRo

aXMNCj4gY29tcHV0ZXIgcHJvZHVjZXMgbGlucmFkLmV4ZS4NCg0KT0suIEkndmUgbm90IHVzZWQg

dGhhdC4NCg0KPj5Eb2VzIHRoYXQgbWVhbiBvbmx5IFJlZGhhdD8NCj4gDQo+IE9vb2ggTm8hISBJ

dCBtZWFucyBldmVyeSAocmVhc29uYWJsZSkgTGludXggZGlzdHJpYnV0aW9uIHRoYXQgaXMNCj4g

bm90IG11Y2ggb2xkZXIgdGhhbiBSZWRIYXQgNi4xLiANCg0KT0suIEkgaGF2ZSBzZWVuIGEgY29t

bWVudCBmcm9tIHNvbWVvbmUgdGhhdCB0aGV5IGFyZSBoYXBweSBpZiB0aGVpciBjb2RlIGNvbXBp

bGVzIG9uIFJlZGhhdCBhbmQgZG8gbm90IA0Kd29ycnkgYWJvdXQgYW55dGhpbmcgZWxzZS4gVGhh

dCBzZWVtcyBhIGJpdCBtdWNoIHRvIG1lIGlmIHlvdSBhcmUgZ29pbmcgdG8gZGlzdHJpYnV0ZSB0

aGUgY29kZSBhcyANCm9wZW4tc291cmNlLg0KDQo+IEkgb2NjYXNpb25hbGx5IHZlcmlmeSB0aGF0

IHRoZSBjb2RlDQo+IHN0aWxsIHJ1bnMgb24gYW4gb2xkIFBlbnRpdW0gY29tcHV0ZXIgd2l0aCB2

ZXJ5IGxpdHRsZSBtZW1vcnkgb24gd2hpY2gNCj4gaXQgaXMgbm90IHNvIGVhc3kgdG8gaW5zdGFs

bCBhIG1vZGVybiBMaW51eCBkaXN0cmlidXRpb24uDQoNClRoYXQgaXMgc2Vuc2libGUuIEkgd3Jv

dGUgYSBzY3JpcHQgZm9yIGRvaW5nIHRoaXMgaW4gYXRsYy4gVGhlIHNjcmlwdCBsb2dzIGludG8g

YQ0KcmVtb3RlIGhvc3QgYnkgcGFzc3dvcmRsZXNzIChwdWJsaWMta2V5L3ByaXZhdGUta2V5KSBz

c2gsIHJ1bnMgdGhlIGNvbmZpZ3VyZSBzY3JpcHQsIGJ1aWxkcyB0aGUgDQpzb2Z0d2FyZSwgcnVu

cyB0aGUgc2VsZiB0ZXN0cyBhbmQgc2VuZHMgYmFjayB0aGUgcmVzdWx0cyAocGFzc2VzIGFuZCBm

YWlsdXJlcykuIEkgYXNzdW1lIHRoYXQgaXMgbm90IA0KdG9vIGVhc3kgdG8gZG8gd2l0aCBMaW5y

YWQsIGFzIHlvdSBuZWVkIHRvIGJlIGF0IHRoZSBtYWNoaW5lIHRvIHRlc3QgaXQuDQoNCj4+QnV0

IG5ldyBhbGdvcml0aG1zIGFyZSBtb3JlIGxpa2VseSB0byBhcHBlYXIgaWYgbW9yZSBwZW9wbGUg

Y2FuIA0KPj5jb21waWxlIGl0LCB3aGljaCBtZWFucyBtYWtpbmcgaXQgbW9yZSANCj4+cG9ydGFi

bGUuIEkgdHJpZWQgY29tcGlsaW5nIGl0IHVuZGVyIFNvbGFyaXMgb24gU1BBUkMgYW5kIHNvb24g

DQo+PnJlYWxpc2VkIGl0IHdhcyB2ZXJ5IExpbnV4IGRlcGVuZGFudC4gSSANCj4+Z2F2ZSB1cCBh

dCB0aGF0IHBvaW50LiBJIGFwcHJlY2FpdGUgdGhlIG51bWJlciBvZiBTUEFSQyB1c2VycyBmb3Ig

DQo+PmhhbSBzb2Z0d2FyZSBtdXN0IGJlIHF1aXRlIGxvdywgYnV0IA0KPj5pZiB0aGUgY29kZSBv

bmx5IGNvbXBpbGVzIG9uIFJlZGhhdCwgeW91IGFyZSBzZXJpb3VzbHkgDQo+PnJlc3RyaWN0aW5n

IHRoZSB1c2VyIGJhc2UuDQo+IA0KPiBTdXJlbHkgdHJ1ZSwgYnV0IHRoYXQgaXMgbm90IHRoZSBj

YXNlLiBJIGhhdmUgdGVzdGVkIHNldmVyYWwgb2YgZWFjaCBvZg0KPiBSZWRIYXQsIE1hbmRyYWtl

LCBTdXNlLCBGZWRvcmEsIERlYmlhbiBhbmQgU2xhY2t3YXJlLiBUaGVyZSBoYXZlIGJlZW4NCj4g

c2V2ZXJhbCBidWdzIGluIHZhcmlvdXMgZGlzdHJpYnV0aW9ucyBhbmQgSSBoYXZlIG1hZGUgdGhl

IGNoYW5nZXMNCj4gcmVxdWlyZWQgdG8gbWFrZSBMaW5yYWQgY29tcGlsZSB1bmRlciBhbGwgb2Yg

dGhlbS4gVGhlcmUgaGF2ZSBhbHNvIGJlZW4NCj4gcXVpdGUgYSBsb3Qgb2YgZXJyb3JzIGluIHRo

ZSBzb3VuZCBzeXN0ZW1zIG92ZXIgdGhlIHllYXJzIGFuZCB0aGVyZWZvcmUNCj4gdGhlIHNvdW5k

Y2FyZCBpbml0IHJvdXRpbmUgaXMgcXVpdGUgYSBiaXQgbW9yZSBjb21wbGljYXRlZCB0aGFuIHdv

dWxkIGJlDQo+IG5lZWRlZCBvbiBhIG1vZGVybiBkaXN0cmlidXRpb24uDQoNCk9LLCB0aGluZ3Mg

YXJlIGNsZWFyIG5vdy4NCg0KPiBUaGUgbWVzc2FnZSBJIHRyaWVkIHRvIGNvbnZleSBpcyB0aGF0

IGl0IGlzIGV4dHJlbWVseSBlYXN5IGZvciBhIG5ld2NvbWVyDQo+IHdpdGggcHJhY3RpY2FsbHkg

bm8gcHJvZ3JhbW1pbmcgZXhwZXJpZW5jZXMgdG8gY2hhbmdlIHRoZSBMaW5yYWQgY29kZS4NCj4g

Q2VydGFpbmx5IGl0IGlzIG5vdCBlYXN5IHRvIGNoYW5nZSB0aGUgYWN0dWFsIHByb2Nlc3Npbmcg

aW4gYSBtZWFuaW5nZnVsDQo+IHdheSwgYnV0IHNlbmRpbmcgY29tbWFuZHMgdG8gYSB0cmFubWl0

dGVyIG92ZXIgdGhlIHNlcmlhbCBwb3J0IG9yIHNob3dpbmcNCj4gc29tZSBtb3JlIGluZm9ybWF0

aW9uIG9uIHRoZSBzY3JlZW4gaXMgdmVyeSBlYXN5LiBFYXNpZXN0LCB3cml0ZSBhIGxpbmUNCj4g

YXQgdGhlIHRvcCBvZiB0aGUgc2NyZWVuICJUSElTIENPUFkgT0YgTElOUkFEIEhBUyBCRUVOIEhB

Q0tFRCBCWSBYWFhYWCIuDQo+IElmIEkgd2VyZSBhIDE0IHllYXJzIG9sZCBuZXdjb21lciBJIG1p

Z2h0IGhhdmUgc3RhcnRlZCBhdCBzb21ldGhpbmcgDQo+IGxpa2UgdGhhdCB0byBnaXZlIHRvIG15

IGZyaWVuZHM7LSkgU2hvd2luZyB0aW1lLCBkYXRlLCBwYXJhbWV0ZXIgdmFsdWVzDQo+IG9yIGFu

eSBvdGhlciBnbG9iYWwgZGF0YSBpcyBhbHNvIHZlcnkgZWFzeS4uLi4NCg0KSSdkIG1pcy1pbnRl

cnByZXRlZCB3aGF0IHlvdSB3ZXJlIHNheWluZy4gU29ycnkgYWJvdXQgdGhhdC4gSXQncyBwcm9i

YWJseSBub3QgaGVscGVkIGJ5IHRoZSBmYWN0IEkgDQpkb24ndCBhY3R1YWxseSBydW4gdGhlIGNv

ZGUuIEkgbG9va2VkIGF0IHVzaW5nIHNvbWUgb2YgaXQgZm9yIGEgcHJvZmVzc2lvbmFsIGFwcGxp

Y2F0aW9uLCBidXQgbmV2ZXIgZGlkLg0KDQotLSANCkRyLiBEYXZpZCBLaXJrYnkgUGhEIENFbmcg

TUlFRSwNClNlbmlvciBSZXNlYXJjaCBGZWxsb3csDQpEZXBhcnRtZW50IG9mIE1lZGljYWwgUGh5

c2ljcywNCk1hbGxldCBQbGFjZSBFbmdpbmVlcmluZyBCdWlsZGluZywNCkdvd2VyIFN0LA0KVW5p

dmVyc2l0eSBDb2xsZWdlIExvbmRvbiwNCkxvbmRvbiBXQzFFIDZCVC4NCg0KDQojIyMjIyMjIyMj

IyMjIyMjIyMjIyMjIyMjIyMjIyMjIyMjIyMjIyMjIyMjIyMjIyMjIyMjIyMjIyMjIyMjDQpUaGlz

IG1lc3NhZ2UgaXMgc2VudCB0byB5b3UgYmVjYXVzZSB5b3UgYXJlIHN1YnNjcmliZWQgdG8NCiAg

dGhlIG1haWxpbmcgbGlzdCA8bGlucmFkQGFudGVubnNwZWNpYWxpc3Rlbi5zZT4uDQpUbyB1bnN1

YnNjcmliZSwgRS1tYWlsIHRvOiA8bGlucmFkLW9mZkBhbnRlbm5zcGVjaWFsaXN0ZW4uc2U+DQpU

byBzd2l0Y2ggdG8gdGhlIERJR0VTVCBtb2RlLCBFLW1haWwgdG8gPGxpbnJhZC1kaWdlc3RAYW50

ZW5uc3BlY2lhbGlzdGVuLnNlPg0KVG8gc3dpdGNoIHRvIHRoZSBJTkRFWCBtb2RlLCBFLW1haWwg

dG8gPGxpbnJhZC1pbmRleEBhbnRlbm5zcGVjaWFsaXN0ZW4uc2U+DQpTZW5kIGFkbWluaXN0cmF0

aXZlIHF1ZXJpZXMgdG8gIDxsaW5yYWQtcmVxdWVzdEBhbnRlbm5zcGVjaWFsaXN0ZW4uc2U+DQoA

ZSAoGwAAKBsAAGxlaWZAc201YnN6LmNvbSB3cm90ZToKPiBIaSBEYXZpZCwKCkggTGVpZiwKCj4+

V2h5IGlzIGl0IHBvc3NpYmxlIHVuZGVyIExpbnV4IG9ubHkgPwo+IAo+IEhtbW0sIHdoYXQgSSBt

ZW50IGlzIHRoYXQgYW4gYXZlcmFnZWx5IHNraWxsZWQgcmFkaW8gYW1hdGV1ciBjYW4KPiBtYWtl

IGNoYW5nZXMgdG8gdGhlIExpbnJhZCBzb3VyY2UgY29kZSwgdHltZSAibWFrZSBsaW5yYWQuZXhl

IiAKPiBhbmQgZ2V0IHNvbWV0aGluZyB0aGF0IHdpbGwgd29yayBvbiB0aGUgV2luZG93cyBtYWNo

aW5lLgoKCk9LLCBJIGdldCB5b3UuIEkgd291bGQgYWdyZWUgYnVpbGRpbmcgY29kZSBvbiBVTklY

IGlzIHZlcnkgZWFzeSAtIGZhciBsZXNzIGhhc3NsZSB0aGFuIG9uIFdpbmRvemUuIFRoZSAKJ2Nv

bmZpZ3VyZScgc2NyaXB0cyB3b3JrIHByZXR0eSB3ZWxsLiBUaGV5IGFyZSBtdWNoIGVhc2llciB0

aGFuIGhhY2tpbmcgTWFrZWZpbGVzLgoKPiBPZiBjb3VyZXMgYW55b25lIHdpdGggYWRlcXVhdGUg

Y29tcHV0aW5nIHNraWxzIGNhbiBkbyB3aGF0ZXZlciAKPiBpdCB0YWtlcyB0byBpbnN0YWxsIHRo

ZSBzb3VyY2UgZmlsZXMsIGEgc3VpdGFibGUgQyBjb21waWxlcgo+IGFuZCBuYXNtIGZvciBNUyBX

aW5kb3dzIGFuZCB0aGVuIHNldCB1cCBhIG1ha2VmaWxlLiAKClN1cmUuCgo+IE9mIGNvdXJzZSBJ

IG11c3QgYWdyZWUgdGhhdCAicG9zc2libGUgdW5kZXIgTGludXgiIG1pZ2h0IGJlCj4gYSBzZXJp

b3VzIG92ZXJzdGF0ZW1lbnQgYnV0IEkgdGhpbmsgeW91IHdpbGwgZmluZCB0aGF0IHRoaXMKPiB3

aWxsIGJlIHNvIGZyb20gYSBwcmFjdGljYWwgcG9pbnQgb2Ygdmlldy4gCgpJIGFtIGdsYWQgdG8g

aGVhciB0aGF0LiBJIGhhdmUgaGVyZCBpcyBzYWlkIHNvbWUgaGF2ZSBpbnN0YWxsZWQgTGludXgg

Zm9yIExpbnJhZCwgd2hpY2ggY2FuIG9ubHkgYmUgYSAKZ29vZCB0aGluZy4gSSBzaG91bGQgYWRk

IEkgZG8gbm90IHJ1biBMaW51eCBtdWNoLiBJIHVzZSBpdCBhdCB3b3JrIG9uIG15IFBDLCBidXQg

YXQgaG9tZSBJIHVzZSAKU29sYXJpcy4gKE1vc3QgcGVvcGxlIHdvdWxkIG5vIGRvdWJ0IGRvIGl0

IHRoZSBvdGhlciB3YXkgYXJvdW5kIC0gU29sYXJpcyBhdCB3b3JrLCBMaW51eCBhdCBob21lISkK

Cj4+RnJvbSBteSBleHBlcmllbmNlIHdpdGggYXRsYywgdGhlcmUgYXJlIHBlcmhhcHMgMTAwIHVz

ZXJzLCBidXQgSSAKPj5zdXNwZWN0IG9ubHkgYSBmZXcgcGVyY2VudCBvZiB0aGVtINmlAQBmeSB0

aGUgY29kZS4KPiAKPiBBIGNvbnNpZGVyIGEgZmV3IHBlcmNlbnQgcXVpdGUgc3VjZXNzZnVsLiAK

Cj4gUHJlc3VtYWJseSB0aGUgcmVhc29ucwo+IGZvciBwZXJzb25hbGl6aW5nIGF0bGMgYXJlIHJh

dGhlciBzbWFsbCBhcyBjb21wYXJlZCB0byBMaW5yYWQuCgpZZXMsIEkgdGhpbmsgdGhhdCBpcyB0

cnVlLgoKPj4+PjEpIElmIHlvdSB3cml0ZSB0ZXh0IGZpbGVzLCB1c2UgZnA9Zm9wZW4oImZvbyIs

InJ0Iikgb3IgCj4+Pgo+PmZvcGVuKCJmb28iLCJ3dCIpLgo+Pgo+Pj4+VU5JWCBzaWxlbnRseSBp

Z25vcmVzIHRoZSAidCIgZm9yIHRleHQgb3IgImIiIGZvciBiaW5hcnksIGJ1dCBmb3IgCj4+Pj5X

aW5kb3dzIGl0IGlzIGltcG9ydGFudC4KPj4+Cj4+PkhtbW0sIEkgYW0gb3BlbmluZyB0aGUgdGV4

dCBma�p##yBsaWtlIHRoaXM6IGZvcGVuKCJmb28iLCJ3Iikgb3IKPj4+Zm9wZW4oImZvbyIsInIi

KS4gSSBoYXZlIG5vdCBub3RpY2VkIGFueSBwcm9ibGVtcyBvdGhlciB0aGFuCj4+PnRoZSB1c3Vh

bCBvbmUgdGhhdCBXaW5kb3dzIHVzZXMgcmV0dXJuIGFuZCBsaW5lLWZlZWQgd2hpbGUgCj4+Pkxp

bnV4IG9ubHkgdXNlcyBvbmUgb2YgdGhlbS4KPj4KPj5JJ3ZlIGNlcnRhaW5seSBzZWVuIHByb2Js

ZW1zLiBJdCBtYXkgaGF2ZSBiZWVuIHdyaXRpbmcgYmluYXJ5IAo+PmZpbGVzLCBJIGNhbid0IHJl

Y2FsbC4gQnV0IEkgdGhpbmsgaXQgaXMgCj4+c2FmZXIgdG8gcHV0IHRoZSAidCIgb3IgImIiLiBU

aGUgc29mdHdhcmUgSSB3cm90ZSBmb3IgCj4+dHJhbnNtaXNzaW9uIGxpbmVzIHdhcyBub3Qgd29y

a2luZyBwcm9wZXJseSBvbiAKPj5ET1MgYW5kIHdhcyBmb3VuZCB0byBiZSB0aGUgbGFjayBvZiB0

aGUgInQiIChvciB3YXMgaXQgImIiID8/KSAKPj5vcHRpb24uIEFkZGluZyBpdCBnb3QgaXQgd29y

a2luZyAKPj5wcm9wZXJseSBvbiBib3RoIHBsYXRmb3Jtcy4KPiAKPiBNYXliZSBpdCBpcyBiZWNh

dXNlIEkgdXNlIHNvbWUgdmVyc2lvbiBvZiBnY2MgdW5kZXIgbWluZ3czMi4KPiBJIGtub3cgdGhh

dCB1bmRlciBET1MgYSBzaW5nbGUgb2NjdXJhbmNlIG9mIGN0bCBaIGlzIGludGVycHJldGVkIAo+

IGFzIGVuZCBvZiBmaWxlIHNvIGEgYmluYXJ5IGZpbGUgaXMgbmV2ZXIgcmVhZCBmdXJ0aGVyIHRo

YW4gdG8KPiB0aGUgZmlyc3Qgb2NjdXJhbmNlIG9mIDI2IGlmIGl0IGlzIG9wZW5lZCBhcyBh